Below is a complete ERP design for a Mill Test Certificate (MTC) Module specifically tailored for Buttweld, Socketweld, Screwed Fittings, Olets, Unions, etc. used in Oil & Gas / Petrochemical supply projects.
This specification is written so your ERP developer can implement the module exactly as required.

# MILL TEST CERTIFICATE (MTC) MODULE – ERP DESIGN

The MTC module consists of three major parts:

1️⃣    MTC Master Configuration
2️⃣    MTC Creation Workflow
3️⃣    MTC Certificate Generation

# 1. MTC MASTER CONFIGURATION

This master controls all specifications and test requirements.

# MATERIAL SPECIFICATION MASTER

Example records:

| Material Spec | Example |
| --- | --- |
| ASTM A234 WPB | Carbon steel BW fittings |
| ASTM A403 WP316L | Stainless steel BW fittings |
| ASTM A105 | Forged fittings |
| ASTM A182 F316L | Stainless forged fittings |

## MASTER FIELDS

| Field | Description |
| --- | --- |
| Material Specification | ASTM grade |
| Starting Material | Pipe / Plate / Round Bar |
| Heat Treatment | Normalized / Solution Annealed |
| Construction Type | Seamless / Welded / Forged |
| Dimension Standard | ASME B16.9 / B16.11 |
| Notes | Default notes |

# CHEMICAL COMPOSITION MASTER

Defines chemical elements for each material specification.

Example for ASTM A234 WPB

| Element | Min | Max |
| --- | --- | --- |
| C | 0 | 0.30 |
| Mn | 0.29 | 1.06 |
| P | 0 | 0.050 |
| S | 0 | 0.058 |
| Si | 0.10 | 0.60 |

Also define result generation range

| Element | Result Min | Result Max |
| --- | --- | --- |
| C | 0.15 | 0.25 |
| Mn | 0.60 | 0.90 |

This range will be used for auto-generated values.

# MECHANICAL PROPERTIES MASTER

Example:

| Property | Min | Max |
| --- | --- | --- |
| Yield Strength | 240 MPa | - |
| Tensile Strength | 415 MPa | 585 MPa |
| Elongation | 22 % | - |
| Reduction Area | 30 % | - |
| Hardness | - | 197 HB |

Also define result generation range.

Example:

| Property | Result Min | Result Max |
| --- | --- | --- |
| Yield Strength | 260 | 310 |
| Tensile Strength | 450 | 520 |

# IMPACT PROPERTIES MASTER

| Parameter | Example |
| --- | --- |
| Test Temperature | -46°C |
| Specimen Size | 10 × 10 mm |
| Minimum Energy | 27 J |

# ADDITIONAL MASTER TABLES

| Master | Example |
| --- | --- |
| Construction Type | Seamless / Welded / Forged |
| Dimension Standard | ASME B16.9 / B16.11 |
| Heat Treatment | Normalized / Quenched |
| Starting Material | Pipe / Plate / Round Bar |

# NOTES MASTER

Example standard notes:

• Material manufactured as per ASTM standards
• Heat treatment carried out as per specification
• Material free from injurious defects

Users can select notes automatically.

# 2. MTC CREATION WORKFLOW

The MTC module should support two types of certificates.

# MTC TYPE SELECTION

User selects:

1) MTC Issued Against PO

2) MTC Issued Against Quotation (Approval)

# WORKFLOW – MTC AGAINST PO

# STEP 1 – BASIC DETAILS

Fields:

| Field | Behavior |
| --- | --- |
| Customer | Autofill from PO |
| PO / PR Number | Select from list |
| PO / PR Date | Autofill |
| Project Name | Autofill (Editable) |
| Certificate No | Auto generated |
| Date | Current date (Editable) |
| Other Reference | User input |

# STEP 2 – ITEM SELECTION

When PO is selected

ERP shows items in that PO.

| Select | Item No | Description | Size | Qty |
| --- | --- | --- | --- | --- |

Rules:

• Maximum 4 items allowed
• All items must have same material specification

System validation:

If material specification differs → selection not allowed

# STEP 3 – MATERIAL DETAILS

User selects:

| Field | Behavior |
| --- | --- |
| Material Specification | User selects |
| Additional Requirement | Optional |
| Starting Material | Autofill |
| Heat Treatment | Autofill |

# STEP 4 – ITEM INFORMATION

Fields auto filled from PO.

| Field | Source |
| --- | --- |
| Item No | PO |
| Description | PO |
| Size | PO |
| Qty | PO |
| Client Item Code | PO |

Fields from master:

| Field | Source |
| --- | --- |
| Construction Type | Material master |
| Dimension Standard | Material master |

User input fields:

Field

Raw Material

Heat No

Heat No can also be auto generated.

# HEAT NUMBER GENERATION

Example format:

H-YYMMDD-XXX

Example:

H-250615-001

# 3. CHEMICAL COMPOSITION SECTION

Auto load elements from material specification master.

Example table:

| Element | Min | Max | H Result | P Result |
| --- | --- | --- | --- | --- |
| C | 0 | 0.30 |  |  |
| Mn | 0.29 | 1.06 |  |  |

# RESULT ENTRY OPTIONS

Two options:

### Option 1 – Manual Entry

User enters values.

### Option 2 – Auto Generate

Button:

Generate Chemical Result

System randomly generates values within defined range.

Example:

| Element | Result |
| --- | --- |
| C | 0.22 |
| Mn | 0.78 |

Each click generates unique combination.

# 4. MECHANICAL PROPERTIES SECTION

Auto populate properties from master.

Example:

| Property | Min | Max | Result |
| --- | --- | --- | --- |
| Yield Strength | 240 | - |  |
| Tensile Strength | 415 | 585 |  |
| Elongation | 22 | - |  |

# SPECIMEN FORM LOGIC

Automatically determined.

### For BW Fittings

If thickness ≥ 19 mm:

Specimen Form = R (Round)

If thickness < 19 mm:

Specimen Form = S (Strip)

# ORIENTATION LOGIC

| Size | Orientation |
| --- | --- |
| Below 8" | L (Longitudinal) |
| 8" and above | T (Transverse) |

# FORGED FITTINGS RULE

For forged fittings:

Specimen Form = R

Orientation = L

# RESULT ENTRY

Options:

Manual entry

or

Auto Generate Result

Values generated within defined range.

# 5. IMPACT PROPERTIES SECTION

Fields auto populated from master.

| Parameter | Value |
| --- | --- |
| Temperature | -46°C |
| Specimen Size | 10x10 mm |

User enters:

Test Result

Impact Energy

# 6. NOTES SECTION

System loads notes from master.

Example:

• Material manufactured as per ASTM specification
• Material free from harmful defects

User can add additional notes manually.

# 7. MTC AGAINST QUOTATION (APPROVAL)

Workflow similar to PO MTC.

Difference:

| Field | Behavior |
| --- | --- |
| Customer | User input |
| Quotation Number | Select |
| Items | Select from quotation |
| Qty | Optional |

Used for client approval before order.

# 8. MTC DOCUMENT GENERATION

ERP should generate standard fixed format MTC.

Sections:

- Certificate Header

- Customer & PO details

- Product description

- Chemical composition

- Mechanical properties

- Impact properties

- Notes

- Authorized signature

# MTC HEADER EXAMPLE

MILL TEST CERTIFICATE

CERTIFICATE NO: MTC-2025-0012

DATE: 15 JUN 2025

# DOCUMENT EXPORT

Formats:

PDF
Print
Email to client

# 9. MTC TRACEABILITY

Each MTC linked to:

Entity

PO

Item

Heat Number

Material Specification

This allows full material traceability.

# 10. MTC DATABASE TABLES

Important tables for developers.

Table

MTC_MASTER

MTC_ITEMS

MTC_CHEMICAL_RESULTS

MTC_MECHANICAL_RESULTS

MTC_IMPACT_RESULTS

MATERIAL_SPEC_MASTER

CHEMICAL_COMPOSITION_MASTER

MECHANICAL_PROPERTIES_MASTER

IMPACT_PROPERTIES_MASTER

MTC_NOTES_MASTER

# 11. MTC DASHBOARD

QA team dashboard.

| Metric | Example |
| --- | --- |
| MTC Created Today | 8 |
| Pending MTC | 3 |
| MTC Approved | 12 |

# 12. OPTIONAL ADVANCED FEATURES

Highly recommended:

### Digital Signature

Sign MTC electronically.

### MTC Number Traceability

Search by:

PO

Heat No

Item

Material Spec

### MTC Revision Control

Track modifications & Edits.

Rev 0 – Original

Rev 1 – Updated chemical results